Libra Reference Board Environment Setup

Availink Inc.

Created: 2012-04-05

Last updated: 2012-04-09

1	Introduction	3

1.1	Purpose	3

1.2	Audience	3

1.3	Assumptions	3

1.4	Glossary	3

2	Release BOM	3

3	App Demo Environment Setup	4

3.1	Overview	4

3.2	Libra EVK Board Connection & Setup	5

3.3	BOOTP/TFTP/NFS Setup	5

3.3.1	BOOTP	6

3.3.2	TFTP Server	6

3.3.3	NFS Server	7

3.4	TFTP/NFS Boot Guide	7

4	Application User Guide	10

4.1	Launching Application	10

4.2	Search Service	11

5	SDK Source Tree Introduction	11

6	Application Build Environment Setup	11

Introduction

Purpose

This document describes the procedure for setting Libra EVK board development environment, including BOOTP/TFTP/NFS server setup, Board and bootloader configutarion, Application user guide, Build system setup (To be added), etc.

Audience

The expected readers are Availink Internal Engineers and engineers from Availink strategic Partners.

Assumptions

The reader should be familiar with the DTV environment as well as Linux system.

Glossary

| Acronym | Meaning |
| --- | --- |

Release BOM

|  | # | Name | Qty | Note |
| --- | --- | --- | --- | --- |
| Hardware | 1 | Libra EVK board | 1 | The main board |
|  | 2 | Libra EVK daughter board | 1 | The tuner board |
|  | 3 | Libra EVK board power supply | 1 | 12VDC |
| Software | 1 | linux-2.6.31.tar.bz2 | 1 | Linux source code (61,494,822 bytes) |
|  | 2 | linux-2.6.31.libra.v.0.0.22.patch.gz | 1 | Libra patch to Linux kernel (7,273,305 bytes) |
|  | 3 | avl_bsp.libra.v.0.0.22.tar.gz | 1 | Libra BSP headers/libraries (22,203,425 bytes) |
|  | 4 | zImage.tz.eth_nosata_usbinkernel | 1 | Kernel image (3,125,440 bytes) |
|  | 5 | rootfs.libra.v.0.0.22-20120410.tar.gz | 1 | Rootfs tar ball (124,103,222 bytes) |
|  | 6 | libra_20120409.tgz | 1 | Libra SDK/App source code (??? bytes) |
|  | 7 | This document | 1 |  |

App Demo Environment Setup

Overview

The most convenient development environment for this evaluation board is using a BOOTP/TFTP/NFS setup. The following diagram shows the overall setup of such an environment.

- The “BOOTP/TFTP/NFS Server”, “Host PC”, and the “Target Board” are all connected to a network Switch/Hub to form a LAN.

- The “Target Board” is also connected to “Host PC” via RS232 cable for debugging purpose.

In cases a separated development network is required (i.e., the EVK board is not connecting the corporate network), the following setup can be used:

- The “BOOTP/TFTP/NFS Server” equipped with two Ethernet interface, one is connected to the corporate network for normal file transfer, and the other is connected to a private subnet for communicating with the EVK boards.

Libra EVK Board Connection & Setup

BOOTP/TFTP/NFS Setup

The following are example configurations on a BOOTP/TFTP/NFS server (Ubuntu 11.10) in a private subnet setup. For other Linux distros, the details may be different.

BOOTP

ae@AEPCSrv:/etc/dhcp3$ /sbin/ifconfig -a eth0 Link encap:Ethernet HWaddr 00:1e:4f:ea:2f:0b inet addr:10.209.156.67 Bcast:10.209.156.255 Mask:255.255.255.0 inet6 addr: fe80::21e:4fff:feea:2f0b/64 Scope:Link UP BROADCAST RUNNING MULTICAST MTU:1500 Metric:1 RX packets:1381841 errors:0 dropped:0 overruns:0 frame:0 TX packets:90069 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:1000 RX bytes:258283874 (258.2 MB) TX bytes:10436917 (10.4 MB) Interrupt:21 Memory:fe9e0000-fea00000 eth1 Link encap:Ethernet HWaddr 84:c9:b2:74:1a:f1 inet addr:192.168.12.34 Bcast:0.0.0.0 Mask:255.255.255.0 inet6 addr: fe80::86c9:b2ff:fe74:1af1/64 Scope:Link UP BROADCAST MULTICAST MTU:1500 Metric:1 RX packets:137349 errors:0 dropped:0 overruns:0 frame:0 TX packets:151103 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:1000 RX bytes:9769271 (9.7 MB) TX bytes:96493826 (96.4 MB) Interrupt:16 Base address:0x8f00 lo Link encap:Local Loopback inet addr:127.0.0.1 Mask:255.0.0.0 inet6 addr: ::1/128 Scope:Host UP LOOPBACK RUNNING MTU:16436 Metric:1 RX packets:192 errors:0 dropped:0 overruns:0 frame:0 TX packets:192 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:0 RX bytes:14188 (14.1 KB) TX bytes:14188 (14.1 KB) ae@AEPCSrv:/etc/dhcp3$ cat dhcpd.conf # # Sample configuration file for ISC dhcpd for Debian # # Attention: If /etc/ltsp/dhcpd.conf exists, that will be used as # configuration file instead of this file. # default-lease-time 600; max-lease-time 7200; ... # This declaration allows BOOTP clients to get dynamic addresses, # which we don't really recommend. subnet 192.168.12.0 netmask 255.255.255.0 { range dynamic-bootp 192.168.12.101 192.168.12.201; range 192.168.12.10 192.168.12.100; } option subnet-mask 255.255.255.0; option broadcast-address 192.168.12.255; option routers 192.168.12.34; ... ae@AEPCSrv:/etc/dhcp3$ sudo /etc/init.d/dhcp3-server restart ...

TFTP Server

ae@AEPCSrv:/etc/default$ cat tftpd-hpa # /etc/default/tftpd-hpa TFTP_USERNAME="tftp" #TFTP_DIRECTORY="/var/lib/tftpboot" TFTP_DIRECTORY="/tftpboot" TFTP_ADDRESS="0.0.0.0:69" TFTP_OPTIONS="--secure" ae@AEPCSrv:/etc/default$ ... mkdir /tftpboot chmod -R 777 /tftpboot service tftpd-hpa restart ...

NFS Server

ae@AEPCSrv:/etc/default$ cat /etc/exports # /etc/exports: the access control list for filesystems which may be exported # to NFS clients. See exports(5). /home *(rw,insecure,async,insecure_locks,no_root_squash) ae@AEPCSrv:/etc/default$ cat /etc/hosts.allow # /etc/hosts.allow: list of hosts that are allowed to access the system. # See the manual pages hosts_access(5) and hosts_options(5). # ae@AEPCSrv:/etc/default$ cat /etc/hosts.deny # /etc/hosts.deny: list of hosts that are _not_ allowed to access the system. # See the manual pages hosts_access(5) and hosts_options(5). # ae@AEPCSrv:/etc/default$ sudo /etc/init.d/nfs-kernel-server restart

TFTP/NFS Boot Guide

- Put kernel image on the folder managed by the TFTP server, for example:

ae@AEPCSrv:/etc/default$ ls /tftpboot/zImage.tz.eth_nosata_usbinkernel_v0.0.22_xiongyw -la -rwxr-xr-x 1 ae ae 3125440 2012-04-09 18:39 /tftpboot/zImage.tz.eth_nosata_usbinkernel_v0.0.22_xiongyw

- Extract the rootfs tar ball (as root) into a folder managed by the NFS server, for example:

ae@AEPCSrv:/home/xiongyw$ mkdir rootfs_v0.0.22 ae@AEPCSrv:/home/xiongyw$ sudo tar xzvf rootfs.libra.v.0.0.22.tar.gz -C rootfs_v0.0.22

Note: extracting the rootfs needs root privileges, otherwise the following error messages will show up:

... tar: dev/vcsa43: Cannot mknod: Operation not permitted tar: dev/mmcblk0p1: Cannot mknod: Operation not permitted tar: dev/tty16: Cannot mknod: Operation not permitted tar: dev/tty0: Cannot mknod: Operation not permitted tar: dev/tty46: Cannot mknod: Operation not permitted tar: dev/tty32: Cannot mknod: Operation not permitted … test_case/lirc_bin_test/irw tar: Exiting with failure status due to previous errors

- Connect the EVK Board to the Host PC via RS232 and configure the Serial port in a serial console application, for example:

- Power on the board, and configure the Bootloader to boot from TFTP/NFS server, for example:

Upon power up, the Bootloader prints the following messages through serial port:

AVL V0.9 NAND(00000022) Availink Done # Time 0: 0: 134, Enter the main of flasher() Libra-1.0.0R Copyright (C) 2011 Availink Co,. ltd. Support: http://www.availink.com Autoboot in progress, press any key to stop...Type "help" to get a list of commands Libra>

Pressing a key to stop the “autoboot” mode and get into the bootloader’s command interface “Libra>”.

Using the following commands to set the necessary variables for a proper TFTP/NFS boot:

/* this is to get the general help for various commands */ # help /* this is to disable the autoboot */ # obd set STOP_AUTO_BOOT 1 /* this is to set the MAC address. Note: the trailing “\00” is needed (same for the reset items below). */ # obd set ETH0_MAC_ADDR \ab\cd\ef\35\71\ae\00 /* this is to set the name of kernel image, which is to be downloaded from tftp server */ # obd set ETH_ZIMAGE_NAME zImage.tz.eth_nosata_usbinkernel_v0.0.22_xiongyw\00 /* this is to set the boot parameters, including the NFS mount path */ # obd set CMD_LINE_ETH_BOOT console=ttyLBR2,115200\20mem=64M\20ip=bootp\20root=/dev/nfs\20rw\20nolock,tcp\20nfsroot=192.168.12.34:/home/xiongyw/rootfs_v0.0.22/\00 /* this is to save the configuration variables */ # obd save /* this is to dump the configuration variables settings */ # obd dump /* this is the kick-off of Ethernet boot via BOOTP/TFTP/NFS protocols */ # doboot eth

The following is a serial log snippet of the boot up messages:

Libra> doboot eth # Time 1049801:1049801:1049935, Enter boot_linux() console=ttyLBR2,115200 mem=64M ip=bootp root=/dev/nfs rw nolock,tcp nfsroot=192.168.12.34:/home/xiongyw/rootfs_v0.0.22/ # Time 15:1049816:1049950, Load Image begin Loading from ETH device... dest is 0x02100000, addrbuf is 0x02100000 synopGMAC_get_phy_id: get_phy_id_1 = 0x22 synopGMAC_get_phy_id: get_phy_id_2 = 0x1513 synopGMAC_check_phy_init: get_phy_id = 0x221513 genphy_config : MII_BMSR = 7849 genphy_config_advert : MII_ADVERTISE = 1e1 genphy_config_advert : adv = 1e1 oldadv = 1e1 genphy_update_link MII_BMSR = 7849 Waiting for PHY auto negotiation to complete . done After config :speed = 100 duplex = 1 link = 1 SynopGMAC: PHY initialized as link-up, 100M, full-duplex. No IP. Bootp start... Our Ethernet address is : AB:CD:EF:35:71:AE Sending bootp packet... . Bootp Packet received. Host (server) Ethernet : 84:C9:B2:74:1A:F1 Host (server) IP : 192.168.12.34 Client (target) Ethernet : AB:CD:EF:35:71:AE Client (target) IP : 192.168.12.102 TFTP Start... Host (server) IP : 192.168.12.34 Client (target) IP : 192.168.12.102 Loading Filename : zImage.tz.eth_nosata_usbinkernel_v0.0.22_xiongyw Save Address : 0x02100000 Loading start... TTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTFTP: 0x00010 TTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTFTP: 0x00020 ... TTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTFTP: complted. 0x002fb0c0(3125440) bytes received. # Time 3748:1053564:1053698, LinuxBoot Begin Starting kernel(0x02100000) ... # Time 8:1053572:1053706, Booting... Uncompressing Linux...................................................................................................................................................................................................... done, booting the kernel. Linux version 2.6.31-svn23390 (yanjt@bjlinux2) (gcc version 4.5.2 (Sourcery G++ Lite 2011.03-41) ) #2 Mon Mar 19 21:28:43 CST 2012 CPU: ARMv6-compatible processor [410fb767] revision 7 (ARMv7), cr=00c5387f CPU: VIPT nonaliasing data cache, VIPT nonaliasing instruction cache Machine: LIBRA SYSTEM BOARD ... SynopGMAC: PHY initialized as link-up, 100M, full-duplex. Sending BOOTP requests . OK IP-Config: Got BOOTP answer from 192.168.12.34, my address is 192.168.12.102 IP-Config: Complete: device=eth0, addr=192.168.12.102, mask=255.255.255.0, gw=192.168.12.34, host=192.168.12.102, domain=, nis-domain=(none), bootserver=192.168.12.34, rootserver=192.168.12.34, rootpath= Looking up port of RPC 100003/2 on 192.168.12.34 Looking up port of RPC 100005/1 on 192.168.12.34 VFS: Mounted root (nfs filesystem) on device 0:10. Freeing init memory: 120K # mount all........... Freeing channel 0 after LPM Freeing channel 0 after LPM # Starting mdev......... ********************************************************* LIBRA Linux-2.6.31 NFS boot ******************************************************** Please press Enter to activate this console. Processing /etc/profile... Done sh-4.1#

At this moment, the TFTP/NFS boot process is successfully done and the board is up, ready for running the demo application.

Application User Guide

This chapter briefly discuss the procedure on how to launch the application from within the serial console, and how to use the remote control to navigate through the menu system, particularly how to do service scan against for example “ChinaSat-6B”.

Launching Application

Following the instruction below to launch the demo application

sh-4.1# cd /test_case sh-4.1# ls -la total 2672 drwxr-xr-x 5 938 1001 4096 Apr 10 2012 . drwxrwxr-x 22 lxin lxin 4096 Apr 10 2012 .. drwxr-xr-x 3 nobody nogroup 4096 Apr 10 2012 application -rwxr--r-- 1 root root 2674639 Apr 10 2012 final_target ... sh-4.1# ./final_target ??? driver init start enter avl_vid_init enter avl_audaudio_open:1300. io_init ^_^:open final_target open /dev/vos sucVOS: ioctl command 2 [0xc00c6502] ceeds VOS: ioctl command 20 [0xc0106514] avl_vos_detect_hd VOS: ioctl command 3 [0xc00c6503] ... AVL/>

At this moment, the video output should look like the following:

Search Service

Blah blah

SDK Source Tree Introduction

Application Build Environment Setup

debian-6.0.2.1-i386-DVD1.iso:

Debian GNU/Linux 6.0.2.1 _Squeeze_ - Offical i386 DVD Binary-1 20110626-15:45

Virutal Box install example:

Debugging…