CLeaReD in Circuits

Created(bruin, 2009-06-24)

Last updated(bruin, 2012-03-01)

- Voltage divider in series circuit: KVL

- Current divider in parallel circuit: KCL

- Bridge circuit

1. Voltage and Current: voltage is the primary concern in analyzing series circuits, where the behavior of the circuit is primarily determined by how the voltage is divided among parts in series.

In parallel circuits, however, since the voltage drops on all branches are the same, the primary concern is how the current is divided among branches in parallel.

http://en.wikibooks.org/wiki/Circuit_Idea/Passive_Voltage-to-Current_Converter

2. Voltage dividing and KVL: in series circuits, all business happened are no more than how the elements divides the voltage imposed among them (they are "worked" togehter either in serial or in parallel, or in a mixed way). put it in a more "human" way, all is about how the elements share the input voltage (pressure, workload, burden, etc). KVL tells that at any time instance (snapshot) of a loop (suppose the loop is the circuit which contains voltage sources), the imposed work load by the sources is equal to the sum of distributed workload.

3. Current dividing and KCL:

4. Ohm's Law: it tells how the work load is distributed among the elements, and/or how the distribution dynamically changes, regardless the structure of the circuit (serial or parallel). this rule is clear and elegant in its mathematical form, that V=Z*I, but it may be a bit difficult to understand (or intuitively feel) in concept, since complex number comes into the scene, i.e., the scalar variables (resistance, and voltage and current) as in DC are extended to vector variables on the complex plane. this extension is “artificial”, in the sense that the imaginary part of voltage (e.g.) is not real and is always discarded in the final result. The benefit of the extension is the unification/elegance in mathematical form, while the price is the requirement of basics knowledge of complex number, not mention the confusion it may cause in the first place. Usually, in text books there is a chapter or an appendix section devoted to the introduction of complex numbers (and usually companied by introduction of calculus). The driving force behind such an anti-intuitive extension is the unique characteristics of C and L (reactance) as compared with R or D (resistance and variable resistance). so a clear understanding of V=Z*I is not only depends on mastering basics of complex numbers, but also, or more importantly, depends on a clear intuitive "feeling" of the characteristics/personalities of C and L.

5. Analogy and personification: it seems, for most of us (i.e., excluding genius), analogy is the best method to achieve that goal, and a lot of efforts have being put into this direction in understanding circuits or electronics. the most prominent one is the hydraulic analogy (e.g., http://en.wikipedia.org/wiki/Hydraulic_analogy), other analogies are mechanical, thermodynamically, etc. while analogies are helpful, they have limits especially when similarities are not available or not applicable, or even contradictions exist. this is inevitable, because the two things under analogy, are, after all, two different things. so multiple analogies may be used in understanding one thing piece-wisely, by keeping awareness of the limits/boundaries of each analogy.

It seems personification, as another approach in understanding things, has less limits in this regard. this is because our human personalities is flexible/versatile as compared to properties of objects. we can shape the personalities when needed while it's not possible to "shape" the property of an object. for a good personification, it's required to "define" the personalities in a reasonable and consistent way, but this is much flexible than choosing properties from objects.

The major approach utilized in this article is personification, aimed as a supplementary approach to various existing analogy approaches.

http://circuit-fantasia.com/circuit_stories/inventing_circuits/virtual_ground/virtual_ground.htm

Using personification instead to explain the idea behind negative resistance and negative feedback: two departments, two work loads (voltage), two set of works (resistors). The virtual ground is the work load boundary between departments, i.e., workers in one department only carry on work load from their own department (thus no affairs between departments). Varying resistance is varying amount of workers, and varying voltage is varying work load.

Treat the short current as the salary standard for the other department: you want the same of salary level? Provide the appropriate work load (from the +/- rails) on your own worker!

6. C: its personality is based on its physical characteristics. C’s capability in sharing voltage burden depends on the strength of its electric field, or the amount of charges on its two plates. Since the amount of charges cannot be changed instantly, its capability in sharing voltage cannot change instantly either. This is known as the general rule that the voltage drop on a capacitor cannot change suddenly.

This can be thought on two sides, that on one side, in responding change/fluctuation of pressure, C is a bit slower. When the input pressure increases, it needs a warm up process (to be charged) for taking more responsibilities, during that time, its colleague (R, or L) will take most of the burden of the pressure. As it been charged, it will take more and more burden of the pressure, thus relieving its colleague in the circuit; One the other side, when the input pressure decreases, it needs a “cooling down” process (to be discharged) to downscale its responsibility. In short, in the sense of taking responsibility in handling pressure fluctuation, as compared to R and L, C is a bit slower but it lasts a bit longer. This is known as C lags to the input voltage. The word “lag” is more often used in describing the relation between I (current through C) and V (the voltage drop on C), but this is obvious (since V is built up by I), and at the same time may be confusing (by introducing current here as an equally important counterpart of voltage). It seems it’s more important or it’s clearer to adhere the meaning of “lag” to the response of input voltage changes (i.e., w/o mentioning current).

C’s voltage formula is also interpreted as C can memorize: it remembers (integrates) the history of current through it (including its initial voltage), and the history is summarized/expressed as the latest voltage drop on it (its memory is no more than that).

C’s capacity can be regarded as the “degree” of its personality, which is proportional; Its impedance can be regarded as depends on both its degree and the frequency of the change.

7. L: its most prominent characteristic is its “inertia”, i.e., it tends to prevent changes in current through it. In contrast to inertia concept in mechanics, there is a physical explanation for L’s inertia: it tends to prevent current change is because it takes time to change the corresponding magnetic field accordingly. This is generally known as the current through L cannot change suddenly.

How L prevents current change lies in how it response to voltage changes in the circuit. It always tries to take full response of the input voltage change in the beginning, this has the effect of preventing changes of burden (voltage drop) taken by its colleagues (e.g., on R, thus the current will not change suddenly). In other words, L is “smart” in preventing current change by trying to keep voltage drop on its colleagues constant. On the other side, it’s not able to maintain the state for long, because the current through it will still change due to the voltage it takes. This can be seen in its current formula as an integration of the history of voltage it buries. It’s more intuitive using mechanical analogy: a=F/m, while F is the voltage, m is the inductance, and a is the change rate of the current through it. So as the current changes, the voltage drops on its colleagues also changes, the net effect is that the burden is gradually shifted from L to its colleagues, thus L is relieved as time goes by. In short, in the sense of taking responsibility of pressure changes, compared to R and C, L is quicker but does not last long. This is also known as L leads the input voltage (at least as compared to R and C, at this moment. More on the meaning of “leads” later on). Similarly, the word “lead” is more often used in describing the relation between V (voltage drop on it) and I (current through it), but this is obvious (since V is the driving force for I), and at the same time may be confusing (by introducing current here as an equally important counterpart of voltage). It seems it’s more important or it’s clearer to adhere the meaning of “lead” in the sense of responding input voltage changes (i.e., w/o mentioning current).

L’s current formula is also interpreted as L can memorize: it remembers (integrates) the history of voltage on it (including its initial current), and the history is summarized/expressed as the latest current through it (its memory is no more than that).

L’s inductance can be regarded as the “degree” of its personality, which is proportional; Its impedance can be regarded as depends on both its degree and the frequency of the change.

8. R: Compared with C and L in responding voltage changes, R is in the middle. It’s “quicker” than C and “slower” than L. It neither proactively takes responsibilities as L, nor defers responsibilities when it comes as C. It takes whatever workload left by its colleagues instantly. This can be regarded as a merit in its personality, but it comes with a cost not seen in C and L: R consumes energy! If we regard the current through R as its income, then R shares work load proportionally according to how much it’s paid! The simplicity of the personality is actually a merit in circuit analysis, that for a known R, the current can be deduced from voltage, vise versa.

A cap in parallel with load: in parallel (with load, bypassing, admittance), or in series (with input impedance, voltage dividing, impedance)? Or both effects exist? The net result is better or worse than a single one? The same issue in FILTERS…

9. How voltage is divided in a series RLC circuit (a), showing both the amplitude and the phase relations, at a particular frequency of Vinput (sine wave).

- Procedure for diagram (b):

- draw the voltage drop on R horizontally;

- prefix voltage drop on L, which leads drop on R in 90°;

- append voltage drop on C, which lags drop on R in 90°;

- connect the tail and head of the line segments, yields the input voltage.

- Amplitude gain: the amplitude of voltage drop is proportional to the length of the vector; the scalar sum of the amplitudes of RLC's voltage drop is greater than or equal to that of input voltage, the sum can be greater is because they are not in phase; It's clear that the maximum drop on R is the input voltage (when drops on L and C are equal in magnitude, i.e., circuit is at resonance), and the maximum drop on L or C can be either less or greater than the input voltage.

- Phase shift: voltage drop on L (and C) leads (and lags) the drop on R by 90°, as current through all of them are in phase with drop on R. Referencing Vinput, VL leads (θ<=90°), VC lags (90°-θ), so VL and VC are always inverse in phase and cancel each other. VR can be either leading or lagging to Vinput, depends on the net sum of VL and VC (i.e., VR leads/lags when the sum is capacitive/inductive, and when the sum is 0, VR is in phase with Vinput).

- Voltage drop on non-pure resistive/reactive impedance is just a vector sum of respective drops, as the dash lines VLR and VCR shown. the total impedance Ztot can be get by summing up the resistive and reactive parts separately (Ztota = Zresistive + jZreactive), as diagram (c) illustrated.

- Why using L and C together in series as their voltage drops cancel each other? first of all, the purpose of a circuit is not to just divide the voltage among elements for fun. Vinput is usually not a single frequency signal, and the prominent purpose of the circuit is to pick out (or filter out) a particular frequency (or ranges of frequencies), by taking the voltage drop on a particular element (could be R, L, R, or their combination, depends on the frequencies we want). actually every frequency components exist in Vinput will be reflected on voltage drops of every elements, one of the differences is that their amplitude gains vary (the other difference is the phase shift). so by picking out (or filtering out) a frequency, it does not mean the other frequencies are disappeared, but just means the amplitude of the desired frequency is bigger enough than that of undesired frequencies (really? how bigger?), or the amplitude gain of that frequency is more than -3dB (in this case, how to make sure that the other frequencies are suppressed)? It also seems VL or VC could be bigger in amplitude than Vinput???

- Frequency response: as f ↑, ZL ↑ and ZC ↓, (and v.v.). Vinput and R do not change, and the 90° phase shift of R/L and R/C are also kept, so the result is that VR/VC/VL, as well as their phase shifts (to Vinput) will all change. So the frequency response (of the voltage drop on a particular element) is to be manifested in two aspects together: amplitude gain and phase shift referencing Vinput.

- Resonance: when |ZL|=|ZC| (by either changing frequency or adjusting L/C values), the sum of their voltage drops is 0 (i.e., totally cancelled out), since the drops are of the same amplitude but inverse in phase. In this case, drop on R reaches its maximum |Vinput| in amplitude (gain=0dB) and it's also in phase with Vinput, yielding the maximum current in the circuit.

10.

- According to Fourier analysis theory, any input voltage signal can be regarded as a combination of sine waves (with different amplitudes and frequencies), so it's sufficient to focus on sine waves when discussing scenarios with AC or non-constant DC signals.

- Ohm's law also applies with ac input voltage (using impedance Z instead of resistance). without considering the phase shifting effect (only consider the amplitude), that means for a sine wave input voltage at a fixed frequency, the percentage of the input voltage that each element is going to carry on is also "fixed", according to the voltage dividing rules (using |Z| of each element instead of resistance). that means the voltage drop on each element is also a sine wave, while the amplitude of that wave is a fixed percentage of the input voltage, the phase may be different.

- Their phases lag/lead to help each other, but on the average, the percentage is fixed.....when a R is working with a C, R needs to lead a bit to compensate C's lag, and in turn, C lags a bit to "compensate" R's lead. so they help each other in different "time zone" in a single circule.

11. personalities/characteristics of RLC:

- For R: given the voltage drop on it, the current through is determined simultaneously. the bigger the voltage drop, the bigger the current through it. the distinguish characteristic of R is that the opposite is also true (deduce voltage from current), and it does not memories. This characteristic is actually a merit, when determining the voltage or current distribution of a circuit: sometimes we know voltage drop on it in the first place, thus we can deduce the current on its path; sometimes we know the current through it in the first place, thus we can deduce the voltage drop on it.

Analogy: V->I: more the pressue (workload), harder it works; I->V: more paid, more workload carried.

This is different from C and L, because:

- For C:

- Given instant V, it's not possible to determine the instant I--we need the change rate of V at this instant; in most cases, instead, we get the instant I from a R which is connected in series with C;

- Given instant I, it's not possible to determine the instant V--we need the history of I, and the initial V; in most cases, instead, we determine the voltage drops of other elements and deduce V over C.

C is slower than R in responsing input voltages,i.e., C's voltage drop lags from the input voltage. when R is in series with C, R needs to be a bit quicker (i.e., leads the voltage) to help C a bit in order to "fight" against the input pressure together. this is not obvious in transicent analysis in the first glance. but considering what lag/lead really means? this not only R is helping C, but C is also helping R in a later stage, i.e., they are helping each other in different time within one period. they are a team!

- For L: given instant V, it's not possible to determine the instant I--we need the history of V and the initial I; given instant I, it's not possible to determine the instant V--we need the change reate of I at this instant;

- For capacitors, its not possible to directly deduce the current through it by giving the voltage (or history of voltage) imposed on it. the current is to be determined by the other type of elements (R or L) in series connection with C, while the

12. The difference (or relation) between transicent analysis and ac (sin wave) analysis?

13. What about Q?