Libra EVK Board Environment Setup

Availink Inc.

Revision History

| Version | Date | Notes |
| --- | --- | --- |
| N/A | 2012-04-05 | Initial creation |
| V0.1 | 2012-04-11 | First release |

1	Introduction	3

1.1	Purpose	3

1.2	Audience	3

1.3	Assumptions	3

2	Release BOM	3

3	App Demo Environment Setup	3

3.1	Overview	3

3.2	Libra EVK Board Layout	5

3.3	BOOTP/TFTP/NFS Setup	5

3.3.1	BOOTP	6

3.3.2	TFTP Server	6

3.3.3	NFS Server	7

3.4	TFTP/NFS Boot Guide	7

3.5	Known Limitations	10

4	Application User Guide	10

4.1	Launching Application	10

4.2	Basic Navigations	11

4.3	Service Scan	12

4.4	Features not Included	13

4.5	Known Issues	13

5	SDK Source Tree Introduction	13

6	Application Build Environment Setup	16

Introduction

Purpose

This document describes the procedure for setting Libra EVK board development environment, including BOOTP/TFTP/NFS server setup, Board and bootloader configutarions, Application user guide, Build system setup (To be added), etc.

Audience

The expected readers are Availink Internal Engineers and engineers from Availink strategic partners.

Assumptions

The reader should be familiar with the DTV environment as well as Linux system.

Release BOM

|  | # | Name | Qty | Note |
| --- | --- | --- | --- | --- |
| Hardware | 1 | Libra EVK main board | 1 | The main board |
|  | 2 | Libra EVK frontend board | 1 | The DVB-S/S2 frontend board |
|  | 3 | Libra EVK board power supply | 1 | 12VDC Power supply |
|  | 4 | Libra EVK RCU | 1 | NEC protocol RCU |
| Software | 1 | zImage.tz.eth_nosata_usbinkernel | 1 | Kernel image (3,125,440 bytes) |
|  | 2 | rootfs.libra.v.0.0.22-20120410.tar.gz | 1 | Rootfs tar ball (124,103,222 bytes) |
|  | 3 | avl_bsp.libra.v.0.0.22.tar.gz | 1 | Libra BSP headers/libraries (22,203,425 bytes) |
|  | 4 | libra_20120409.tgz | 1 | Libra SDK & App source code (4,165,714 bytes) |
|  | 5 | This document | 1 |  |

App Demo Environment Setup

Overview

The most convenient development environment for this evaluation board is using a BOOTP/TFTP/NFS setup. The following diagram shows the overall setup of such an environment.

- The “BOOTP/TFTP/NFS Server”, “Host PC”, and the “Target Board” are all connected to a network Switch/Hub to form a LAN.

- The “Target Board” is also connected to “Host PC” via RS232 cable for debugging purpose.

In cases a separated development network is required (i.e., the EVK board is not connecting the corporate network), the following setup can be used:

- The “BOOTP/TFTP/NFS Server” equipped with two Ethernet interface, one is connected to the corporate network for normal file transfer, and the other is connected to a private subnet for communicating with the EVK boards.

Libra EVK Board Layout

BOOTP/TFTP/NFS Setup

The following are example configurations on a BOOTP/TFTP/NFS server (Ubuntu 11.10) in a private subnet setup. For other Linux distros, the details may be different.

BOOTP

ae@AEPCSrv:/etc/dhcp3$ /sbin/ifconfig -a eth0 Link encap:Ethernet HWaddr 00:1e:4f:ea:2f:0b inet addr:10.209.156.67 Bcast:10.209.156.255 Mask:255.255.255.0 inet6 addr: fe80::21e:4fff:feea:2f0b/64 Scope:Link UP BROADCAST RUNNING MULTICAST MTU:1500 Metric:1 RX packets:1381841 errors:0 dropped:0 overruns:0 frame:0 TX packets:90069 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:1000 RX bytes:258283874 (258.2 MB) TX bytes:10436917 (10.4 MB) Interrupt:21 Memory:fe9e0000-fea00000 eth1 Link encap:Ethernet HWaddr 84:c9:b2:74:1a:f1 inet addr:192.168.12.34 Bcast:0.0.0.0 Mask:255.255.255.0 inet6 addr: fe80::86c9:b2ff:fe74:1af1/64 Scope:Link UP BROADCAST MULTICAST MTU:1500 Metric:1 RX packets:137349 errors:0 dropped:0 overruns:0 frame:0 TX packets:151103 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:1000 RX bytes:9769271 (9.7 MB) TX bytes:96493826 (96.4 MB) Interrupt:16 Base address:0x8f00 lo Link encap:Local Loopback inet addr:127.0.0.1 Mask:255.0.0.0 inet6 addr: ::1/128 Scope:Host UP LOOPBACK RUNNING MTU:16436 Metric:1 RX packets:192 errors:0 dropped:0 overruns:0 frame:0 TX packets:192 errors:0 dropped:0 overruns:0 carrier:0 collisions:0 txqueuelen:0 RX bytes:14188 (14.1 KB) TX bytes:14188 (14.1 KB) ae@AEPCSrv:/etc/dhcp3$ cat dhcpd.conf # # Sample configuration file for ISC dhcpd for Debian # # Attention: If /etc/ltsp/dhcpd.conf exists, that will be used as # configuration file instead of this file. # default-lease-time 600; max-lease-time 7200; ... # This declaration allows BOOTP clients to get dynamic addresses, # which we don't really recommend. subnet 192.168.12.0 netmask 255.255.255.0 { range dynamic-bootp 192.168.12.101 192.168.12.201; range 192.168.12.10 192.168.12.100; } option subnet-mask 255.255.255.0; option broadcast-address 192.168.12.255; option routers 192.168.12.34; ... ae@AEPCSrv:/etc/dhcp3$ sudo /etc/init.d/dhcp3-server restart ...

TFTP Server

ae@AEPCSrv:/etc/default$ cat tftpd-hpa # /etc/default/tftpd-hpa TFTP_USERNAME="tftp" #TFTP_DIRECTORY="/var/lib/tftpboot" TFTP_DIRECTORY="/tftpboot" TFTP_ADDRESS="0.0.0.0:69" TFTP_OPTIONS="--secure" ae@AEPCSrv:/etc/default$ ... mkdir /tftpboot chmod -R 777 /tftpboot service tftpd-hpa restart ...

NFS Server

ae@AEPCSrv:/etc/default$ cat /etc/exports # /etc/exports: the access control list for filesystems which may be exported # to NFS clients. See exports(5). /home *(rw,insecure,async,insecure_locks,no_root_squash) ae@AEPCSrv:/etc/default$ cat /etc/hosts.allow # /etc/hosts.allow: list of hosts that are allowed to access the system. # See the manual pages hosts_access(5) and hosts_options(5). # ae@AEPCSrv:/etc/default$ cat /etc/hosts.deny # /etc/hosts.deny: list of hosts that are _not_ allowed to access the system. # See the manual pages hosts_access(5) and hosts_options(5). # ae@AEPCSrv:/etc/default$ sudo /etc/init.d/nfs-kernel-server restart

TFTP/NFS Boot Guide

- Put kernel image on the folder managed by the TFTP server, for example:

ae@AEPCSrv:/etc/default$ ls /tftpboot/zImage.tz.eth_nosata_usbinkernel_v0.0.22_xiongyw -la -rwxr-xr-x 1 ae ae 3125440 2012-04-09 18:39 /tftpboot/zImage.tz.eth_nosata_usbinkernel_v0.0.22_xiongyw

- Extract the rootfs tar ball (as root) into a folder managed by the NFS server, for example:

ae@AEPCSrv:/home/xiongyw$ mkdir rootfs_v0.0.22 ae@AEPCSrv:/home/xiongyw$ sudo tar xzvf rootfs.libra.v.0.0.22.tar.gz -C rootfs_v0.0.22

Note: extracting the rootfs needs root privileges, otherwise the following error messages will show up:

... tar: dev/vcsa43: Cannot mknod: Operation not permitted tar: dev/mmcblk0p1: Cannot mknod: Operation not permitted tar: dev/tty16: Cannot mknod: Operation not permitted tar: dev/tty0: Cannot mknod: Operation not permitted tar: dev/tty46: Cannot mknod: Operation not permitted tar: dev/tty32: Cannot mknod: Operation not permitted … test_case/lirc_bin_test/irw tar: Exiting with failure status due to previous errors

- Connect the EVK Board to the Host PC via RS232 and configure the Serial port in a serial console application, for example:

- Power on the board, and configure the Bootloader to boot from TFTP/NFS server, for example:

Upon power up, the Bootloader prints the following messages through serial port:

AVL V0.9 NAND(00000022) Availink Done # Time 0: 0: 134, Enter the main of flasher() Libra-1.0.0R Copyright (C) 2011 Availink Co,. ltd. Support: http://www.availink.com Autoboot in progress, press any key to stop...Type "help" to get a list of commands Libra>

Pressing a key to stop the “autoboot” mode and get into the bootloader’s command interface “Libra>”.

Using the following commands to set the necessary variables for a proper TFTP/NFS boot:

/* this is to get the general help for various commands */ # help /* this is to disable the autoboot */ # obd set STOP_AUTO_BOOT 1 /* this is to set the MAC address. Note: the trailing “\00” is needed (same for the reset items below). */ # obd set ETH0_MAC_ADDR \ab\cd\ef\35\71\ae\00 /* this is to set the name of kernel image, which is to be downloaded from tftp server */ # obd set ETH_ZIMAGE_NAME zImage.tz.eth_nosata_usbinkernel_v0.0.22_xiongyw\00 /* this is to set the boot parameters, including the NFS mount path */ # obd set CMD_LINE_ETH_BOOT console=ttyLBR2,115200\20mem=64M\20ip=bootp\20root=/dev/nfs\20rw\20nolock,tcp\20nfsroot=192.168.12.34:/home/xiongyw/rootfs_v0.0.22/\00 /* this is to save the configuration variables */ # obd save /* this is to dump the configuration variables settings */ # obd dump /* this is the kick-off of Ethernet boot via BOOTP/TFTP/NFS protocols */ # doboot eth

The following is a serial log snippet of the boot up messages:

Libra> doboot eth # Time 1049801:1049801:1049935, Enter boot_linux() console=ttyLBR2,115200 mem=64M ip=bootp root=/dev/nfs rw nolock,tcp nfsroot=192.168.12.34:/home/xiongyw/rootfs_v0.0.22/ # Time 15:1049816:1049950, Load Image begin Loading from ETH device... dest is 0x02100000, addrbuf is 0x02100000 synopGMAC_get_phy_id: get_phy_id_1 = 0x22 synopGMAC_get_phy_id: get_phy_id_2 = 0x1513 synopGMAC_check_phy_init: get_phy_id = 0x221513 genphy_config : MII_BMSR = 7849 genphy_config_advert : MII_ADVERTISE = 1e1 genphy_config_advert : adv = 1e1 oldadv = 1e1 genphy_update_link MII_BMSR = 7849 Waiting for PHY auto negotiation to complete . done After config :speed = 100 duplex = 1 link = 1 SynopGMAC: PHY initialized as link-up, 100M, full-duplex. No IP. Bootp start... Our Ethernet address is : AB:CD:EF:35:71:AE Sending bootp packet... . Bootp Packet received. Host (server) Ethernet : 84:C9:B2:74:1A:F1 Host (server) IP : 192.168.12.34 Client (target) Ethernet : AB:CD:EF:35:71:AE Client (target) IP : 192.168.12.102 TFTP Start... Host (server) IP : 192.168.12.34 Client (target) IP : 192.168.12.102 Loading Filename : zImage.tz.eth_nosata_usbinkernel_v0.0.22_xiongyw Save Address : 0x02100000 Loading start... TTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTFTP: 0x00010 TTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTFTP: 0x00020 ... TTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTTFTP: complted. 0x002fb0c0(3125440) bytes received. # Time 3748:1053564:1053698, LinuxBoot Begin Starting kernel(0x02100000) ... # Time 8:1053572:1053706, Booting... Uncompressing Linux...................................................................................................................................................................................................... done, booting the kernel. Linux version 2.6.31-svn23390 (yanjt@bjlinux2) (gcc version 4.5.2 (Sourcery G++ Lite 2011.03-41) ) #2 Mon Mar 19 21:28:43 CST 2012 CPU: ARMv6-compatible processor [410fb767] revision 7 (ARMv7), cr=00c5387f CPU: VIPT nonaliasing data cache, VIPT nonaliasing instruction cache Machine: LIBRA SYSTEM BOARD ... SynopGMAC: PHY initialized as link-up, 100M, full-duplex. Sending BOOTP requests . OK IP-Config: Got BOOTP answer from 192.168.12.34, my address is 192.168.12.102 IP-Config: Complete: device=eth0, addr=192.168.12.102, mask=255.255.255.0, gw=192.168.12.34, host=192.168.12.102, domain=, nis-domain=(none), bootserver=192.168.12.34, rootserver=192.168.12.34, rootpath= Looking up port of RPC 100003/2 on 192.168.12.34 Looking up port of RPC 100005/1 on 192.168.12.34 VFS: Mounted root (nfs filesystem) on device 0:10. Freeing init memory: 120K # mount all........... Freeing channel 0 after LPM Freeing channel 0 after LPM # Starting mdev......... ********************************************************* LIBRA Linux-2.6.31 NFS boot ******************************************************** Please press Enter to activate this console. Processing /etc/profile... Done sh-4.1#

At this moment, the TFTP/NFS boot process is successfully done and the board is up, ready for running the demo application.

Known Limitations

The current release of kernel/rootfs does not enable telnetd/sshd. So the only way to access the system is through the serial console.

Application User Guide

This chapter briefly discuss the procedure on how to launch the application from within the serial console, and how to use the remote control to navigate through the menu system, particularly how to do service scan against for example “ChinaSat-6B”. It’s not intended to include a full-blown Application User Guide (for end-users) here.

Launching Application

Following the instruction below to launch the demo application:

sh-4.1# cd /test_case sh-4.1# ls -la total 2672 drwxr-xr-x 5 938 1001 4096 Apr 10 2012 . drwxrwxr-x 22 lxin lxin 4096 Apr 10 2012 .. drwxr-xr-x 3 nobody nogroup 4096 Apr 10 2012 application -rwxr--r-- 1 root root 2674639 Apr 10 2012 final_target ... sh-4.1# ./final_target ??? driver init start enter avl_vid_init enter avl_audaudio_open:1300. io_init ^_^:open final_target open /dev/vos sucVOS: ioctl command 2 [0xc00c6502] ceeds VOS: ioctl command 20 [0xc0106514] avl_vos_detect_hd VOS: ioctl command 3 [0xc00c6503] ... AVL/>

Note: as shown above, the demo application resides in “/test_case” directory. The executable is “final_target”, and the “application” directory contains the GUI resource/assets used by the executable.

At this moment, the screen output is the main menu, which looks like the following:

Basic Navigations

The diagram below is the button layout of the RCU.

The main menu system of the application is arranged into 2 levels, represented by the two “rainbows” on the left side of the screen.

- The left-most “rainbow” is the 1st level, and the 2nd “rainbow” is the 2nd level.

- Pressing the “Menu” key toggles the focus between 1st level menu and the live video;

- To move the focus from the 1st level menu items to the 2nd level menu items, press “Ok” button.

- To move the focus back from the 2nd level menu items to the 1st level menu items, press either “Back” or “Menu”.

- To invoke function from the 2nd menu item, press “Ok” button. A dialog will show up in the right side of the screen. The focus also moves to the dialog.

- For going back to Live video screen (from 1st/2nd menu levels), press “Exit” button.

Service Scan

This section briefly describes how to manually scan service on a transponder, take “ChinaSat-6B” TP 30# as an example.

- Make sure to connect the live signal to the F-connector of the NIM on the tuner board. Be note that the top F-connector is “RF In”, the bottom F-connector is “RF Loop Out”.

- Press “Menu” to enter the 1st level menu and navigate up-down to “TV Manager” item.

- Press “Ok” to enter the 2nd level menu and navigate up-down to “Program Search” item.

- Press “Ok” to invoke the service scan dialog, which contains several tabs.

- To move focus among dialog tabs: first move the focus (up-down) to the “Search” button, and then move the tab focus by left and right buttons. Let’s focus on the “Manual Search” tab.

- Within the “Manual Search” tab, using up-down button to move the focus to first item “Satellite”, and using left-right button to scroll among all built-in satellite names. Let’s select ChinaSat-6B (note that the satellite name displayed is in Simplified Chinese “中星6B”, regardless of the current language settings).

- Move to the 2nd item “TP” and left-right scrolling to locate on TP 30#. During the TP scrolling, the built-in lock parameters will also change accordingly. ChinaSat-6B TP30# has the following lock parameters: frequency (3840), symbol rate (27500), Polarization (Horizontal). After selecting the TP, the application will try to lock the TP in the background, and if it’s locked, the signal “Qaulity” and “Strength” bars (at the bottom of the tab) will be activated with dynamic live data.

- Choose a “Search Mode” as desired, the default is “All”. Other choices are “Free”, “TV”, and “Radio”.

- Move the focus to “Search” button and press Ok button. The manual search progress dialog will show up, and after successful service scan, the application goes into live TV mode by zapping to one of the service it just scanned.

Features not Included

The following features are not included in this release:

- Media Player

- PVR

When highlighting the related sub-menus and pressing the “Ok” button, nothing will happen.

Known Issues

Besides the possible stability issues, the current release has the following known issues:

- Restarting Application: once started, the application cannot gracefully exit the system and restart properly. One needs to reset the board and reboot the system again for restarting the application.

- Debugging Application: the GDB multiple-treading debugging is not working properly, this makes the GDB of little use for the application (which is a multi-threads process).

SDK Source Tree Introduction

The following table briefly describes the Availink SDK and Application source code directories as included in this release. Be noted that the directory structure as well as the contents within each directory are subject to change for later releases.

| Modules | Directory Tree | Description |
| --- | --- | --- |
|  | libra_20120409/ | Root directory |
| main() | ¦-- main | The entry point of the application |
| Demo Application | ¦-- application |  |
|  | ¦ `-- libra_hdtv | Application with 720p UI |
|  | ¦ ¦-- epg |  |
|  | ¦ ¦-- include |  |
|  | ¦ ¦-- main |  |
|  | ¦ ¦-- misc |  |
|  | ¦ `-- res | GUI resources |
|  | ¦ ¦-- font |  |
|  | ¦ `-- image |  |
| SDK | ¦-- middleware |  |
|  | ¦ ¦-- dvb | DVB stack |
|  | ¦ ¦ ¦-- database | Database for various metadata |
|  | ¦ ¦ ¦ `-- FileSystem | File system API for database |
|  | ¦ ¦ ¦-- manager | Application state machine manager |
|  | ¦ ¦ ¦-- parser | DVB parser |
|  | ¦ ¦ `-- service | DVB services |
|  | ¦ ¦ ¦-- hangup | Dummy |
|  | ¦ ¦ ¦-- player | Main component for play Video/Audio from TS |
|  | ¦ ¦ `-- search | Service scan |
|  | ¦ ¦-- facility |  |
|  | ¦ ¦ ¦-- frontend | Tuner and demod |
|  | ¦ ¦ ¦-- interface | Front panel, IR, keyboard |
|  | ¦ ¦ ¦-- nvram | NVRAM |
|  | ¦ ¦ `-- transfer | Demux section filtering |
|  | ¦ ¦-- include |  |
|  | ¦ ¦ ¦-- dvb |  |
|  | ¦ ¦ ¦-- facility |  |
|  | ¦ ¦ ¦ `-- gui |  |
|  | ¦ ¦ ¦-- librawidget |  |
|  | ¦ ¦ ¦-- manager |  |
|  | ¦ ¦ ¦-- misc |  |
|  | ¦ ¦ ¦-- monitor |  |
|  | ¦ ¦ ¦-- oc |  |
|  | ¦ ¦ ¦-- service |  |
|  | ¦ ¦ `-- terminal |  |
|  | ¦ ¦-- librawidget | Widgets used in application |
|  | ¦ ¦ `-- theme |  |
|  | ¦ ¦-- misc |  |
|  | ¦ ¦-- monitor | Monitor for system status and PSI/SI |
|  | ¦ ¦-- terminal |  |
|  | ¦ ¦ ¦-- console | Console utility |
|  | ¦ ¦ ¦ `-- test |  |
|  | ¦ ¦ `-- log | Log utility |
|  | ¦ `-- thirdpart |  |
|  | ¦-- bsp |  |
|  | ¦ ¦-- peripherals | Board level BSP |
|  | ¦ ¦ ¦-- demod |  |
|  | ¦ ¦ ¦ `-- avl_dvbs_plus |  |
|  | ¦ ¦ ¦-- flash |  |
|  | ¦ ¦ ¦-- front_panel |  |
|  | ¦ ¦ ¦-- include |  |
|  | ¦ ¦ ¦ ¦-- demod |  |
|  | ¦ ¦ ¦ ¦-- flash |  |
|  | ¦ ¦ ¦ ¦-- front_panel |  |
|  | ¦ ¦ ¦ `-- tuner |  |
|  | ¦ ¦ `-- tuner |  |
|  | ¦ ¦ `-- Sharp |  |
|  | ¦ `-- platform | SoC level BSP include and libraries |
|  | ¦ ¦-- include |  |
|  | ¦ ¦ ¦-- driver |  |
|  | ¦ ¦ `-- sys |  |
|  | ¦ `-- libra |  |
|  | ¦ ¦-- driver |  |
|  | ¦ `-- sys |  |
| Headers | ¦-- include |  |
|  | ¦ `-- config |  |
|  | ¦ ¦-- app |  |
|  | ¦ ¦ `-- final |  |
|  | ¦ ¦-- avl |  |
|  | ¦ ¦ `-- cfg |  |
|  | ¦ ¦-- cas |  |
|  | ¦ ¦-- dbc |  |
|  | ¦ ¦-- gui |  |
|  | ¦ ¦ ¦-- driver |  |
|  | ¦ ¦ `-- lib |  |
|  | ¦ ¦-- host |  |
|  | ¦ ¦ `-- buildroot |  |
|  | ¦ ¦-- libra |  |
|  | ¦ ¦ `-- os |  |
|  | ¦ ¦-- platform |  |
|  | ¦ `-- terminal |  |

Application Build Environment Setup

This part is not included in this release yet.